Lifts

Double Scissors lift – under body type

- Lifting height min 1900 mm

- Lifting capacity min 3000kg

- Platform length min 1400 mm max 1900 mm

- The base of the Scissors lift shall not exceed 1900 mm in length as it will interfere with the mechanics area to move on.

Lift - Two Post

- Lifting height, minimum 2000mm

- Lifting capacity, minimum 3000kg

- Drive-through width, minimum 2500mm

See attachment for picture

Preferred

- Without beam across the floor between pillars

## Lift - Single Post in ground

- Lifting height, minimum 1400mm

- Lifting capacity, minimum 3000kg

- Width, minimum 1700mm

See attachment for picture.

Recommendation

- With possibility for vehicle anchoring to the H-formed construction.

- Smaller alignment jobs.

- To be used with Body Straightener.

## Scissors lift

- Lifting height, minimum 1900mm

- Lifting capacity, minimum 3000kg

- Platform length, minimum 1400mm max 1900

See attachment for picture

Lift - Two Post - Asymmetric

- Lifting height min 1900 mm

- Lifting capacity 3000 kg

- Drive-through width min 2500 mm

- Lifting points:

- Length from 1475mm to 1950mm

- Width from 900mm to 1350mm

Preferred

- Without beam across floor (allows clear floor)

Lift - Two Post - Symmetric

- Lifting height min 1900 mm

- Lifting capacity 3000 kg

- Drive-through width min 2500 mm

- Lifting points:

- Length from 1475mm to 1950mm

- Width from 900mm to 1350mm

Preferred

- Asymmetric lift makes it easier to work on the car.

Four Post Lift

- Lifting capacity min 3000 kg

- Platform length min 4400 mm

- Platform width min 450 mm

- Lift width - inside posts min 2600 mm

Recommendation:

Options

- Front recesses for turning plates - wheel alignment / testing

- Rear slip plates - wheel alignment

- Scissors jacks - wheel alignment

- Lifting capacity min 2000 kg

- Width lifting arm 1000-1550 mm

Scissors Lift - Platform type

- Lifting capacity from bottom position 3000 kg

- Lifting height min 1900 mm

- Platform length min 4150 mm

- Platform width min 450 mm

Recommendations

Options

- Front recesses for turning plates - wheel alignment / testing

- Rear slip plates

- Scissors jacks

- Lifting capacity 2000 kg

- Width lifting arm 1000-1400

Universal Component Lift

- Lifting capacity min 500 kg at available air pressure, if air powered

- Lifting height at platform:

- Lower position max 900mm

- Upper position min 1700 mm

Recommendation:

Options

- Platform size min 400 mm x 400 mm

- Adapter kit for Volvo Cars

Exhaust extractor

- Hose length 15m

See attachment for picture

Recommendation:

- Twin extractor for vehicles with twin pipes.

Brake Tester - roller type Type A: 2 wheel drive or 4 wheel drive with AOC (Active On demand Coupling) system

Notice! From model year 2009 on it is important the maximum testing time 1 minute and maximum speed of 5 km/h not are exceeded, otherwise it will cause over heating.

Alternatively the not tested axle is running on free rolling rollers like for cars with VISCO-coupling as described below.This does not apply for previous year models with Haldex AOC-system.

- Brake power 10 - 6000 N

- Axle load min 1200 kg

- Rollers fringe:

min 2080mm  max 1080mm

- Note: For 4 wheel drive brake test should be done with haldex-fuse out or ignition switch OFF. Type B: 4 wheel drive with VISCO system

- Brake power 10 - 6000 N

- Axle load min 1200 kg

- Rollers fringe:

min 2080mm      max 1080mm

- One of following two requirements must be fulfilled:

- 1. The brake tester must have a system that can test the brakes without moving power to the other axle, for example the brake tester have a control unit that keeps wheel rolling with exactly the same speed, one wheel clockwise and one wheel counter clockwise.
Notice: For P2-cars with VISCO system you can test front and back wheels like this. For X70-cars (X70 is cars up to year 2000) only the brakes in the front wheels can be tested using this type of equipment.

- 2. This applies for car models -2002 except for S60*

- 3. Only front wheel brakes can be tested with this equipment.

- 4. The brake tester must have free rolling roller for the axle that is not under test.

Additional requirements (concerns the two types above):

- In addition to the brake power the display shall also be able to show the brake power difference in % right/left.

- Recommendation (concerns the two types above):  connected to printer

Wheel Alignment - 4 wheel

Measuring range Tolerance range

- Total front Toe ± 2° ± 0,03°

- Individual front Toe ± 2° ± 0,02°

- Camber front ± 8° ± 0,03°

- Camber rear ± 8° ± 0,03°

- Caster ± 19° ± 0,08°

- SAI (KPI) ± 19° ± 0,08°

- Setback ± 2° ± 0,04°

- Total rear Toe ± 2° ± 0,04°

- Individual rear Toe ± 2° ± 0,03°

- Thrust angle ± 2° ± 0,02°

- Colour printer

Additional requirements

- The sensors shall manage the compensation of distortion with 3-points compensation or rolling compensation (AWD) and also manage wheels with rim diameters up to 21".

Tyre Changer

- Tightening a wheel from outside and/or inside 13" - 21" (330 mm - 533 mm)

- Rim width 3,5" - 12" (90 mm - 305 mm)

- Tire width 155 - 300 mm

- Wheel & tyre diameter 850 mm

Additional requirements

- Force required to brake bead 7,5 kN (at available air pressure, if air powered)

- Torque require to remove tyre 1000 Nm

Recommendation:

- The Tyre changer should be supplied with plastic protection to avoid damage on aluminium rims.

Wheel Balancer

- Rim width 3,5 - 12" (90 mm - 305 mm)

- Rim diameter 13 - 21" (330 mm - 533 mm)

- Tire width 155 - 300 mm

- Tyre diameter min 850 mm

- Wheel weight min 40 kg

- Balance accuracy min ±1g

Recommendation:

- The wheels should be able to be centred through the rims centre hole with attachment diameter 55 - 75 mm.

- Hidden weight facility

- Face plate attachment

Fuel Tank Drainer

- Tank volume minimum 80 litres

- Suction capacity minimum 7 litres/minute
(when pick-up hose outer diameter maximum 11,5 mm)

- Pick-up hose length minimum 1900 mm

- Pick-up hose material Flexible steel

- Pick-up hose outer diameter maximum 11,5 mm

- Earth ground cable length minimum 2500 mm

- Water separation feature

- Flame protection is an absolute must when handling ethanol

Recommendations

- The equipment should have security valve for closing the suction system from overfilling the vessel.

- The equipment should have a safety valve for overpressure in vessel if air powered.

- The equipment should have fuel level indicator.

- The hoses should have quick-couplings, that are closed when not connected.

- When the tank drainer is not in use it should be completely sealed.

- The equipment should be easy to move.

- The equipment should have holders for the hoses.

- The equipment should have holders for the cables.

- It should be possible to connect a needle valve of diameter 7 mm to the equipment.

- It should be possible to connect a connector with inner dimension M12x1.5. Purpose: Connect Volvo Special Tool No. 9995479.

- It should be possible to connect a hose with inner diameter 6 mm. Purpose: Connect Volvo Special Tool No. 9995480.

Straightening / pulling equipment

- Pulling force min 5 ton at a height of min 700 mm above the lower edge of the sill.

- Possibility to have higher attachment points for the pulling point on the vehicle. Of a minimum height of 1500 mm above the lower edge of the sill at an angle of 45 degrees from the vehicle.

- Possibility to fix 3 individual pulling points on the front and one on the rear of the vehicle at the same time.

- Possibilities to fix one pulling point at the front, rear and side of the vehicle at the same time.

- Possibility to measure high measurement points on the vehicle at a height of a minimum of 1500 mm above the lower edge of the sill.

- Access to Volvo measurement data and continuous updates.

For drive on equipment

- Lifting capacity min 3000 kg

- Platform length min 4150 mm

- Platform width min 450 mm

Suppliers with access to Volvo measurement data are:

- Celette

- Car-O-Liner

- Car Bench

- Spanesi

Dryer Booth Owen

Box

- Interior dimension, length minimum 8000mm.

- Interior dimension, width minimum 4000mm.

- Interior dimension, height minimum 2700mm.

- Lattice floor load, minimum 3500kg.

- Lattice floor load per wheel, minimum 800kg.

- Should have one safety door minimum.

See attachment for picture

Temperature

- Drying Temperature, approximately 65°C / 149°F.

Air flow

- It should be the same exhaust air flow over whole floor space.

Filters

- Intake filter efficiency 45%.

- Distance between floor exhaust filters and lattice floor, minimum 50mm
(To avoid that the floor filters pollutes the air in the booth).

- Filter change, Should be installed with alarm system.

Fan ventilator system

- Air overpressure, maximum 10Pa.

Combi booth

- During bake process, exchange booth air with new air, minimum 20% 
(of booth air volume).

Recommendations

- Inside and outside noise level, maximum 65dBA

- The Spray Booth door opening width should be the same as interior width.

Paint mixing room

Area

- Interior dimension, length minimum 4600 mm.

- Interior dimension, width minimum 2500 mm.

- Interior dimension, height minimum 2700 mm.

- Should have two safety door minimum.

See attachment for picture.

Temperature

- Temperature, approximately 22°C.

Air flow

- Stand by ventilation 800 m3/h.

- Day time 3000 m3/h.

- Working ventilation 6000 m3/h.

- Spark free fan with separate ventilation system.

- Filter change, Should be install with alarm device.

Fan ventilator system

- Air overpressure, maximum 10 Pa.

- Air negative pressure not allowed.

Recommendations

- Inside and outside noise level, maximum 65dba.

- Mixing Booth should be integrated with the Spray Booth and have minimum 2 doors. One door to the Spray Booth and one door to the workshop.

Paint preparation station

Area

- Interior dimension, length minimum 8000 mm

- Interior dimension, width minimum 6000 mm

- Interior dimension, height minimum 3000 mm

- Lighting, approximately 1500 lux

See attachment for picture

Fan ventilator system

- Air overpressure, maximum 10 Pa

- Air negative pressure not allowed

- Stand by ventilation, approximately 1500 m3/h

- Working ventilation, approximately 3000 m3/h

- Ventilation should be controlled by timer

Recommendations

- Inside and outside noise level, maximum 65dba

- Should be equipped with a lift

- Should have an IRT panel system installed

Plastic Mixing Cabinet

- Airflow, approximately 400m³/h

- Width 600mm

- Height 700mm

- Depth 500mm

Separate Air Extraction

See attached picture

Airline hose

- Hose, recommended length 15m

See attachment for picture.

Battery Charger - for use on the car, in the workshop

Power Supply for SWDL

- Voltage maximum 14.2 V

- Nominal current, minimum 30 A

Support Charger

- Voltage maximum, when charging the battery 14.2 V

- Nominal current, minimum 30 A

- Automatic resume full charging effect before below 75 % charge.

Show Room Charger

- Voltage maximum, when charging the battery 14.2 V

- Nominal current, minimum 10 A

- Automatic resume full charging effect before below 75 % charge.

Maintenance Charger

- Voltage maximum, if no fume cupboard 14.2 V 
(when charging the battery)

- Nominal current, minimum 5 A

Storage Charger

- Voltage maximum 14.2 V

- Nominal current, minimum 1 A

Quick Charger

- 60 % state of charge within 60 minutes without gasing.

Comment:

This specification is applicable for battery charger that is used in the workshop or on the car. Therefore the charging voltage is specified to maximum 14,2 V to avoid gazing. When a fume cupboard is used for charging a battery, the charging voltage can be higher than 14,2 V.

Glass repair Tool

- Pump capacity, vacuum - 0.9 bar

- Pump capacity, pressure 1 bar

- Gauge to measure vacuum and pressure

- Chemical must comply with list of forbidden substances in:

- DSL-list

- TSCA-list

- KECL-list

- EINECS-list

- Or other national lists

Recommendations

- Manual in local language.

- Containing safety glasses.

Information

The gauge is necessary in order to verify that there is sufficient vacuum and pressure. Correct vacuum is vital in order to have water and moister to evaporate at 20° Celsius.

Spot Welder

A cars body is part of the safety system and therefore it is of high importance that the equipment can perform high class welds. Volvo approved spot welders have gone through comprehensive tests at Volvo Cars or independent institutes.

The approved spot welders are the only that are allowed to be used on Volvo cars and according to Volvo standard.

Approved spot welders are:

| Product | Manufacturer/Supplier |
| --- | --- |
| Tecna 3650 | Tecna |
| Elektron Mi100 | Elektron |
| Car-o-liner CR500 | Car-o-liner |
| Celette MIDI-SPOT 9000 | Celette |
| MIDISpot QSVM 9000 | Elmatech |
| InvertaSpot ATL | Wielander |
| ProSpot PR 2000 | Prospot |
| Alia Technik | HySpot 20 |
| Gyspot 125I | GYS |
| Telwin Invertspotter 1400 | Telwin |

Volvo aftermarket test procedure approved.

On all welders listed there have been several tests performed, both on several different steel qualities and different combinations.

Spot welding gun.

Together with the approved spot welder a spot welder gun that fulfills the specification must be used.

Specification:

P1: Working depth, minimum 300mm

P2: Electrode opening, minimum 100mm

P3: Electrode pressure, minimum 240daN

Body Straightener

- Pulling force minimum 5 ton.

- Flexibility beam minimum 45 degrees.

See attached picture as an example.

Benefits

Use for smaller damages .

Very useful together with H head one pillar lift for smaller repairs.

Spray Booth (Combi box)

Box

- Interior dimension, length minimum 8000 mm.

- Interior dimension, width minimum 4000 mm.

- Interior dimension, height minimum 2700 mm.

- Lattice floor load, minimum 3500 kg.

- Lattice floor load per wheel, minimum 800 kg.

- Lightning Zone 2 minimum 1500 Lux.

See attachment for picture.

Temperature

- Temperature variation when painting, maximum +/- 1°C.

- Dryer temperature, approximately 65°C / 149°F

Air flow

- Airflow sink speed, minimum 0,2 m/s.

- It should be the same exhaust air flow over whole floor space.

- There should be no air turbulence in the booth working-zone.

Recycling

- Energy recycling 80%.

Filters

- Intake filter efficiency 45%.

- Distance between floor exhaust filters and lattice floor, minimum 50 mm.
(To avoid that the floor filters pollutes the air in the booth).

- Filter change, Should be install with alarm device.

Fan ventilator system

- Air overpressure, maximum 10 Pa.

- Air negative pressure not allowed.

Combi booth

- During bake process, exchange booth air with new air, minimum 20%.

Recommendations

- Inside and outside noise level, maximum 65dBA

- Mixing Booth should be integrated with the Spray Booth and have minimum 2 doors. One door to the Spray Booth and one door to the workshop.

- The Spray Booth should manage that you spray many colours at the same time.

- When the Spray Booth is installed in colder climes the Spray Booth should have throttle that prevents cold air in intake air channel and exhaust air channel.

- The Spray Booth door opening width should be the same as interior width.

- For the spray gun hose, there should be 2 suspensions placed diagonally, to avoid that the spray gun hose drags on the floor.

- If Combi Booth: When the bake period starts the Spray Booth should automatically be ventilated for 180 seconds before hot air.

- Capacity for Combi Booth is five to seven object per/ eight hours period.

- With IRT frame system you can add two more objects.

- Combi box can be Heated with, electrical power / central heating / oil / gas.

- Compare different energy sources and dry time per object for best cost efficiency.

Tie REF page 1

Tie Ref Page 2